MCT-454L: Mobile Robotics

Autonomous Soccer Agent: Motion Modeling and Control in ROS 2

Session: 2022

SUBMITTED TO

Dr. Maria Akram

SUBMITTED BY

Muhammad Haider Ali		2022-MC-45

Department of Mechatronics & Control Engineering, University of Engineering & Technology, Lahore, Pakistan

# I. System Architecture

The control system is structured as a modular ROS 2 package composed of four Python modules, each with a single well-defined responsibility. This separation of concerns makes each component independently testable and easy to modify.

| Module | Role | Key Exports |
| --- | --- | --- |
| control.py | Top-level ROS 2 node; state machine; vision callback; timer loop | BallTrackerNode |
| apf.py | Artificial Potential Field planner — attractive + repulsive + tangential forces | compute_apf() |
| utils.py | Math helpers — angle normalization, clamping, frame transform | normalize_angle(), world_to_robot() |
| packet.py | grSim UDP encoder (protobuf-like, no external dependency) | build_packet() |
| main.py | Entry point — rclpy init, spin, shutdown | main() |

The node subscribes to /ssl_vision_bridge/vision_messages (VisionWrapper) at 50 Hz and sends UDP velocity commands directly to grSim on port 20011. The control loop runs on a 20 ms timer, decoupled from the vision callback, so sensor latency does not stall the controller.

## Data Flow

Vision data arrives via the ROS 2 topic and is parsed in _vision_cb(). Ball position, robot pose (x, y, yaw extracted from quaternion), and all opponent and teammate positions are stored as instance state. Obstacle positions are time-stamped and expired after 1 second if not refreshed, preventing ghost obstacles from stale frames. The control loop then reads this shared state every 20 ms, computes velocity commands through the active state handler, and dispatches a UDP packet.

## State Machine

The striker operates as a three-state finite state machine:

- APPROACH_BALL — navigate to the ball using APF + PD damping; hold for 25 ticks (~0.5 s) before transitioning.

- ALIGN — arc behind the ball onto the shoot axis, then rotate to face the goal; declare READY after 10 stable ticks within 0.10 rad angular tolerance.

- READY — drive into the ball with kicker armed at 6.0 m/s; reset to APPROACH_BALL once the ball departs.

Each state includes a ball-moved guard: if the ball relocates beyond the threshold while in ALIGN or READY, the machine resets immediately to APPROACH_BALL, preventing the robot from executing a kick on a ball it has lost.

# II. Motion Model

## Holonomic Kinematics

The robot is omni-directional (holonomic), meaning it can translate in any direction independently of its heading. Commands are sent in the robot's local frame as three scalars: veltangent (forward), velnormal (lateral left), and velangular (CCW rotation). World-frame velocity vectors from the planner are converted to robot frame using a standard 2D rotation:

vt =  cos(−θ)·vx − sin(−θ)·vy

vn =  sin(−θ)·vx + cos(−θ)·vy

where θ is the robot's current yaw from the vision system. This transform is implemented in world_to_robot() in utils.py.

## PD Control — Angular

Heading is controlled by a PD controller. The proportional term drives the robot toward the desired angle; the derivative term suppresses overshoot by opposing rapid angular change:

ω = Kp_ang · e_θ + Kd_ang · (Δe_θ / Δt)

Gains are Kp = 4.0, Kd = 0.05, with output clamped to ±6.0 rad/s. Angular error is always normalized to [−π, π] using atan2(sin(e), cos(e)) before computing the derivative, preventing derivative spikes at the ±π wrap boundary.

## PD Control — Translational

The APF output serves as the proportional term for translational velocity. A derivative term is added using the rate of change of distance error to damp overshoot near the ball:

v_pd = v_apf + Kd_vel · (Δdist / Δt)

When the robot is closing in, Δdist/Δt is negative, reducing speed automatically. The PD-adjusted speed is clamped to [0, 2.5 m/s] and re-applied along the original APF direction vector, so only the magnitude is modified, not the obstacle-avoidance heading.

# III. Planning Strategy

## Artificial Potential Fields (APF)

Navigation uses an Artificial Potential Field approach. The robot is subject to an attractive force toward the target and repulsive forces from nearby obstacles. These are summed and normalized to produce a world-frame velocity command.

| Component | Formula | Behaviour |
| --- | --- | --- |
| Attractive (near) | F_att = K_att · (target − robot) | Linear pull when dist ≤ D_star (1.0 m) |
| Attractive (far) | F_att = K_att · D_star · unit(target − robot) | Capped pull at distance; prevents runaway speed |
| Repulsive | F_rep = K_rep · (1/ρ − 1/ρ₀) / ρ² · unit(away) | Pushes robot away when within ρ₀ = 0.28 m |
| Tangential | F_tan = K_tan · mag · dot · tangent | Sidesteps robot around obstacle toward goal |

Up to N=7 nearest obstacles are considered per tick. Each obstacle's effective radius is reduced by 0.18 m (robot body radius) so the influence field reflects surface-to-surface distance. Repulsive force magnitude is capped at 20.0 to prevent numerical blowup at near-zero distance. The tangential component adds a lateral nudge that steers the robot around obstacles rather than purely away from them, reducing the likelihood of getting trapped in saddle points.

## Behind-Ball Arc Maneuver

During ALIGN, the robot must position itself behind the ball on the ball-to-goal axis before facing the goal and kicking. If the robot is not yet behind the ball, a tangential arc waypoint is computed: the robot-from-ball unit vector is rotated 90° in whichever direction reduces the angle to the behind-ball target. The robot is sent to this arc waypoint at half speed (1.25 m/s), skirting around the ball without pushing it. Once the behind-ball check passes, the robot drives to the hold position at ALIGN_DIST = 0.22 m behind the ball.

## Why Not ORCA?

Optimal Reciprocal Collision Avoidance (ORCA) was evaluated as a candidate for dynamic obstacle handling. ORCA offers provably collision-free paths in multi-agent environments by computing velocity half-planes for each agent pair and finding a velocity inside the intersection. However, several practical barriers made it unsuitable for this project:

- Velocity estimation required: ORCA needs reliable velocity estimates for all other agents. In the SSL vision pipeline, robot velocities must be differentiated from noisy position data, introducing lag and noise that destabilize ORCA constraints.

- Reciprocity assumption broken: ORCA assumes all agents run the same algorithm and split avoidance responsibility equally. Opponent robots in grSim do not run ORCA, so the robot must take the full avoidance burden — requiring a conservative safety margin that over-constrains the feasible velocity set.

- Solver complexity: A proper ORCA implementation requires a linear programming solver (or half-plane intersection) at each control tick per agent pair, adding implementation complexity well beyond the course scope.

- Out of course scope: Only potential fields were covered in the curriculum. ORCA is a research-grade algorithm from the UNC GAMMA group, and implementing it correctly under time pressure introduces more risk than benefit.

The APF approach with tangential forces handles the observable obstacle avoidance cases in the SSL environment adequately. The main limitation — local minima when obstacles cluster between robot and ball — is mitigated by the tangential nudge and the arc-around-ball maneuver during ALIGN, which uses a geometrically constructed waypoint rather than relying on the APF to avoid the ball itself.

# 4. Edge Cases and Robustness

| Edge Case | Detection | Response |
| --- | --- | --- |
| Ball moves during ALIGN | dist > 2 × STOP_DIST | Immediate reset to APPROACH_BALL |
| Ball moves after READY | dist > 2 × STOP_DIST | Abort kick, reset to APPROACH_BALL |
| No vision data | self._ball or self._robot is None | Control loop skips tick, logs warning |
| Stale obstacle data | Timestamp expiry > 1 s | Obstacle removed from map |
| Duplicate vision frames | dt < 5 ms between frames | Ball velocity update skipped |
| Angle wrap at ±π | normalize_angle() on all errors | atan2(sin,cos) prevents derivative spike |
| Near-zero obstacle distance | rho = max(rr − 0.18, 0.01) | Clamp prevents division by zero |
| APF local minimum | Tangential force + arc waypoint | Lateral push escapes saddle points |
| Overshoot near ball | Translational D term (Kd=0.15) | Speed reduced as distance closes |
| Robot not behind ball | Dot product with shoot axis | Arc maneuver reroutes around ball |

## Ball Velocity Estimation

Ball velocity is tracked using an Exponential Moving Average (EMA) with smoothing factor α = 0.4. Raw velocity is computed as finite difference of ball position over time. Frames with dt < 5 ms are skipped to avoid division by near-zero time differences producing spurious large velocity estimates. The smoothed velocity is stored but in the current implementation is available for future use (e.g., intercept prediction) — the state machine uses measured ball position directly.

## Obstacle Map Lifecycle

All detected robots (opponents and teammates excluding self) are stored in a dictionary keyed by robot ID with a timestamp. Every vision callback prunes entries older than 1 second. This prevents phantom obstacles from robots that have left the field or stopped being detected, which would otherwise bias the APF repulsive field in incorrect directions.

# 5. Results and Limitations

## Observed Performance

In simulation testing with grSim, the striker successfully:

- Navigated from any starting position to the ball using APF within 3–5 seconds depending on distance.

- Executed the arc-around-ball maneuver to position behind the ball on the goal axis.

- Aligned heading to the goal within 0.10 rad tolerance before kicking.

- Scored goals consistently when no defender was present, and in most trials with a static defender.

## Known Limitations

- Dynamic defenders moving directly into the robot's path can temporarily trap the APF in a local minimum before the tangential force resolves it. Increasing RHO_0 (influence radius) mitigates this at the cost of wider detours.

- The ALIGN arc maneuver can be slow when the robot starts directly in front of the ball (opposite side from goal), requiring a near-180° arc. A larger ALIGN_DIST would reduce arc time but risks pushing the ball on approach.

- Ball velocity is estimated but not yet used for predictive interception. Adding lead compensation would improve performance against a moving ball.

## Parameter Summary

| Parameter | Value | Purpose |
| --- | --- | --- |
| MAX_VEL | 2.5 m/s | Maximum translational speed |
| MAX_OMEGA | 6.0 rad/s | Maximum angular speed |
| STOP_DIST | 0.15 m | Proximity threshold to ball |
| ALIGN_DIST | 0.22 m | Hold distance behind ball |
| KP_ANG / KD_ANG | 4.0 / 0.05 | Angular PD gains |
| K_ATT / K_REP | 4.0 / 6.0 | APF attractive/repulsive gains |
| RHO_0 | 0.28 m | Obstacle influence radius |
| K_TAN | 3.0 | Tangential sidestep gain |
| KICK_SPEED | 6.0 m/s | grSim kick speed |
| HOLD_TICKS / ALIGN_TICKS | 25 / 10 | Stability dwell counters |

## References

grSim Simulator: https://github.com/RoboCup-SSL/grSim

ssl_ros_bridge: https://github.com/SSL-A-Team/ssl_ros_bridge

Khatib, O. (1986). Real-time obstacle avoidance for manipulators and mobile robots. International Journal of Robotics Research, 5(1), 90–98.

van den Berg, J. et al. (2008). Reciprocal velocity obstacles for real-time multi-agent navigation. ICRA 2008.